TEMPORARY BARRIER RAIL [618] (REVISED 1-15-26)

Description. This work includes furnishing, installing, maintaining, and removing temporary barrier rail used for lane separation or protection of construction work zones from traffic.

Materials. Furnish concrete barrier rail in accordance with Section 605 and the Detailed Drawings. Alternative barrier design meeting NCHRP 350 TL-3 or MASH TL-3 crash testing may be used. Submit proof of crash testing compliance 14 calendar days prior to installation.

Construction Requirements. Furnish, install, maintain, and remove temporary barriers as required by the contract and manufacturer’s instructions. Terminate the ends of temporary concrete barrier with a temporary impact attenuator or with approved blunt end protection as stated in the contract. Blunt end protection not shown on the plans is at the Contractor’s expense.

Place 3” x 4” retroreflective markers every 10’ on the top of the temporary rail. Chip Seal tabs and raised rigid pavement markers are prohibited. Ensure reflective markers are in place on existing barrier rail and/or guardrail adjacent to temporary rail.

When required by the contract, anchor the barrier in accordance with the Detailed Drawings.

Method of Measurement.

Temporary Barrier Rail. Temporary barrier rail will be measured by the foot of barrier placed to protect or channelize traffic as specified in the plans. Transition pieces are incidental to the barrier rail bid item. Temporary terminal end protection will be measured for payment. Loading the barrier on a trailer or transport to move it to a new location or set up will be paid for as a new installation.

Reset Temporary Barrier Rail. Reset temporary barrier rail is measured by the foot and includes the complete reinstallation and removal of a temporary barrier that was previously installed and moved to a new installation or location. Using equipment or manpower to push, pull, lift, drag, or otherwise move the barrier rail to a new location or set up is defined as a reset.

Temporary and Reset Barrier Rail – Anchored. When required by the contract, anchored barrier rail is measured by the linear foot of barrier placed.

Temporary Impact Attenuator. Temporary end protection is measured in accordance with Subsection 606.04.8.

Temporary removal and resetting of barrier to protect the same obstacle will not be measured for payment.

Installation or resetting of barrier rail required by the Contractor’s operations and not shown on plans, or the repair or replacement of temporary barrier rail damaged by the Contractor, will not be measured for payment. Removal and replacement of installations damaged by traffic will be measured for payment.

Basis of Payment. Payment for the completed and accepted quantities is made under the following:

| Pay Item | Pay Unit |
| --- | --- |
| Temp Barrier Rail | LNFT |
| Reset Temp Barrier Rail | LNFT |
| Temp Barrier Rail - Anchored | LNFT |
| Reset Temp Barrier Rail – Anchored | LNFT |