electronic ticketing – PMS [401] (added 1-15-26)

Description. This work consists of providing electronic ticketing (E- ticket) for material delivery of plant mix surfacing to the project.

Equipment and Programming. Provide a programming interface to integrate with the Department’s E-ticketing portal Haulhub. Utilize the interface to provide electronic data from the load read-out weigh system to provide information as described in Subsection 401.03.8(B).

The Department’s E-ticket portal can be found at the following link:

MDT E-Ticket Portal

Internet Availability. E-ticketing requires internet access. Ensure internet access at the hot plant location.

Set-up and Calibration. A minimum of 3 calendar days prior to plant mix paving, set-up and calibrate the interface with the Department’s Haulhub portal and provide at least 5 calibration E-tickets for plant mix weights marking each ticket “Test”.

Upload time. Ensure the upload time to the Department’s portal does not exceed 5 minutes from the time the ticket was created.

Representatives from Haulhub will be available virtually to assist in the set-up, calibration, and production throughout the project duration. Contact information for Haulhub can be found at the following link: https://www.haulhub.com/agency/montana

For immediate support contact Carlos Osorio at Haulhub; Phone (929) 298-3396; email: carlos.osorio@haulhub.com

Printed tickets will be required along with E-tickets unless otherwise directed by the Project Manager.

All work for programming, training and utilizing E-ticketing is not measured for payment.